TFLite

On-device sign inference for ESP32-P4 with TensorFlow Lite Micro. Captures frames from an IMX219 or OV5647 camera, runs the B-G colour-difference pipeline, classifies the sign, and sends the result to the ESP32-S3 over UART.

# Camera Selection

Edit image_provider.h:

#define CAMERA_TYPE CAMERA_TYPE_IMX219   // or CAMERA_TYPE_OV5647 /

## CAMERA_TYPE_AUTO

IMX219: MIPI CSI, 1536×1232 RAW10. Library builds at IMG_SIZE (96×96 default) via the override in image_provider.h.

OV5647: MIPI CSI, 1920×1080 RAW10. The library stays at its own 160×160 default —

GetImage() resizes to IMG_SIZE at runtime.

AUTO: probes the shared I2C bus at startup (IMX219=0x10, OV5647=0x36) and picks whichever responds — no recompile needed when swapping cameras.

Both cameras feed the same pipeline, so switching cameras only requires changing CAMERA_TYPE.

# Preprocessing Mode

Edit image_provider.h:

#

define

PREPROCESS_MODE PREPROCESS_MODE_BG

// or GRAY / RGB

#

define

PREPROCESS_MODE PREPROCESS_MODE_BG

// or GRAY / RGB

| Mode | What GetImage produces | Use case |
| --- | --- | --- |
| PREPROCESS_MODE_BG (default) | B-G colour difference + blob auto-crop + contrast | Sign inference (matches AItraining/image_preprocess.py) |
| PREPROCESS_MODE_GRAY | Plain BT.601 grayscale + contrast, no colour difference | Inference on models trained with plain grayscale |

No inference — streams

WB-corrected

Data collection for AItraining, like the

PREPROCESS_MODE_RGB	IMG_SIZE×IMG_SIZE×3

IMX219_RGB_Serial example

RGB to Serial (0xAA 0x55

0xAA + payload)

# Pipeline

Camera → dynamic Gray-World AWB → B-G extraction → blur → contrast stretch → blob detection → auto-crop → resize IMG_SIZE×IMG_SIZE → contrast → TFLite int8 inference → UART to S3.

See the repo-root CLAUDE.md for the full 13-step pipeline. Any change to image_provider.cpp must be mirrored in AItraining/image_preprocess.py.

# UART Protocol (P4 ↔ S3, bidirectional)

Both directions share one UART at 921600 baud (P4 pins: RX=10, TX=11).

P4 → S3 — inference packet, 9 bytes:

## Offset	Field

| 0-1 | Sync 0xAA 0x55 |
| --- | --- |
| 2 | msg_type 0x01 |
| 3-4 | frame_id (uint16 LE) |
| 5 | label_id |
| 6 | confidence |
| 7 | flags (2 = sign_ready) |

8	checksum (XOR of bytes 0-7)

S3 → P4 — control packet, 5 bytes:

## Offset	Field

| 0-1 | Sync 0xAA 0x55 |
| --- | --- |
| 2 | msg_type 0x02 |
| 3 | command |

4	checksum (XOR of bytes 0-3)

Commands:

0x01ACK_STOP — S3 confirmed the sign → P4 stops transmitting (inference continues)

0x02RESUME — S3 done → P4 resumes transmitting

TX gating is a volatile bool s_transmit_enabled; when disabled the TX task drains its queue without writing. GPIO10 RX noise is mitigated with INPUT_PULLDOWN.

# FreeRTOS Tasks

| Task | Core | Priority | Stack | Role |
| --- | --- | --- | --- | --- |
| tflm | 1 | 3 | 32 KB | Capture → pipeline → inference → queue packet |
| Task | Core | Priority | Stack | Role |
| uart_tx | 0 | 2 | 4 KB | Send packets; drops when s_transmit_enabled == false |
| uart_rx | 0 | 2 | 4 KB | Read S3 control packets; toggle s_transmit_enabled |

sd	0	1	8 KB	Write PGM frames to SD

# Configurable Parameters

| Param | Default | Description |
| --- | --- | --- |
| CAMERA_TYPE | IMX219 | CAMERA_TYPE_IMX219 / _OV5647 / _AUTO (I2C probe) |
| PREPROCESS_MODE | BG | PREPROCESS_MODE_BG / _GRAY / _RGB |
| kDebugBaud | 921600 | Debug serial baud |
| kUartBaud | 921600 | P4↔S3 UART baud |
| kEnableSdLogger | true | Save frames to SD |

kSaveEveryNFrames 10	Save every N frames